SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- EXPERIMENTS

| Course Code: | CSA1205 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO3 | Assessment: | ASSESSMENT TOOL 1- EXPERIMENTS |
| Weightage: | 40% | Total Marks: | 25 |
| CO3 Statement: | Design processor organization by constructing pipelined data path structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). (BL6) |  |  |
| Student Name: | S Mohammed Kaif | Reg. No.: | 192311259 |
| Department: | CSE | Date: | 17-08-2026 |

ANNEXURE A

1. Design and simulate a system that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR. Use multiple registers to demonstrate how data is transferred between them during execution, and analyze the sequence of operations and the role of the ALU in processing data.

2. Create a model of a computer system using single-bus and multi-bus organization, and perform data transfer operations between registers, memory, and I/O. Compare the time taken and efficiency of both approaches, and analyze how multiple bus structures improve parallel data transfer and reduce bottlenecks.

3. Simulate a 5-stage instruction pipeline (Fetch, Decode, Execute, Memory, Write-back) and execute a set of instructions. Measure the performance in terms of speedup and throughput, and compare it with a non-pipelined system to evaluate the effectiveness of pipelining.

4. Develop a pipeline execution scenario where data hazards, control hazards, and structural hazards occur. Observe their impact on instruction execution, and implement techniques such as stalling, data forwarding, and branch prediction to resolve these hazards and improve performance.

5. Analyze a processor supporting superscalar execution, including out-of-order and speculative execution, and extend the study to include hyper-threading and simultaneous multithreading (SMT). Evaluate how multiple instructions are executed in parallel and compare the performance improvements in terms of instruction throughput and resource utilization.

Code of Conduct:

I S. mohammed Kaif (192311259) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Understanding of Concepts (20%) | Experimental Design (20%) | Use of Tools (25%) | Analysis of Results (20%) | Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 - Exemplary | Level 3 - Proficient | Level 2 - Developing | Level 1 - Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |

EXPERIMENT 1 – REGISTER TRANSFER OPERATIONS AND THE ROLE OF THE ALU

Aim: To design and simulate a system that performs register transfer operations along with basic arithmetic and logic operations (addition, subtraction, AND, OR) using multiple registers, and to analyze the sequence of operations and the role of the ALU in processing data.

Program:

#include <stdio.h> typedef struct { char name[4]; int value; } Register; int ALU(int a, int b, char op) { switch (op) { case '+': return a + b; case '-': return a - b; case '&': return a & b; case '¦': return a ¦ b; default: return 0; } } void printReg(Register r) { printf("%s = %d\n", r.name, r.value); } int main() { Register R1 = {"R1", 18}; Register R2 = {"R2", 7}; Register R3 = {"R3", 0}; Register R4 = {"R4", 0}; printf("Initial Register Contents:\n"); printReg(R1); printReg(R2); R3.value = R1.value; // Register Transfer R1 -> R3 printf("\nTransfer R1 -> R3\n"); printReg(R3); R4.value = ALU(R1.value, R2.value, '+'); printf("\nR4 = R1 + R2 -> "); printReg(R4); R4.value = ALU(R1.value, R2.value, '-'); printf("R4 = R1 - R2 -> "); printReg(R4); R4.value = ALU(R1.value, R2.value, '&'); printf("R4 = R1 AND R2 -> "); printReg(R4); R4.value = ALU(R1.value, R2.value, '¦'); printf("R4 = R1 OR R2 -> "); printReg(R4); return 0; }

Result:

Initial Register Contents: R1 = 18 R2 = 7 Transfer R1 -> R3 R3 = 18 R4 = R1 + R2 -> R4 = 25 R4 = R1 - R2 -> R4 = 11 R4 = R1 AND R2 -> R4 = 2 R4 = R1 OR R2 -> R4 = 23

Analysis:

The sequence begins with a register transfer, where the contents of R1 are copied onto the common data path and loaded into R3 without any modification, showing that transfer operations simply move data between storage elements under control-signal timing. The ALU then takes the values of R1 and R2 as inputs and performs addition, subtraction, AND, and OR based on the operation code supplied by the control unit, placing each result in R4. This demonstrates that the ALU is the functional unit responsible for all arithmetic and logical computation in the datapath, while registers act purely as temporary storage that feed operands to, and capture results from, the ALU. Correct sequencing (transfer before compute, compute before store) is essential, since operating on stale register values would produce incorrect results.

EXPERIMENT 2 – SINGLE-BUS AND MULTI-BUS ORGANIZATION

Aim: To create a model of a computer system using single-bus and multi-bus organization, perform data transfer operations between registers, memory, and I/O, and compare the time taken and efficiency of both approaches.

Program:

#include <stdio.h> int main() { int transfers = 6; // register/memory/I-O transfers needed int singleBusCycles = transfers; // 1 transfer per cycle int multiBusCycles = (transfers + 2) / 3; // 3 buses -> 3 transfers/cycle float singleEff = (float)transfers / singleBusCycles; float multiEff = (float)transfers / multiBusCycles; printf("SINGLE-BUS ORGANIZATION\n"); printf("Total transfers = %d, Cycles required = %d\n", transfers, singleBusCycles); printf("Efficiency = %.2f transfers/cycle\n\n", singleEff); printf("MULTI-BUS ORGANIZATION (3 buses)\n"); printf("Total transfers = %d, Cycles required = %d\n", transfers, multiBusCycles); printf("Efficiency = %.2f transfers/cycle\n", multiEff); return 0; }

Result:

SINGLE-BUS ORGANIZATION Total transfers = 6, Cycles required = 6 Efficiency = 1.00 transfers/cycle MULTI-BUS ORGANIZATION (3 buses) Total transfers = 6, Cycles required = 2 Efficiency = 3.00 transfers/cycle

Analysis:

In the single-bus organization every transfer between registers, memory, and I/O must share one common path, so only one transfer can occur per clock cycle; six transfers therefore require six cycles and the bus utilization stays at exactly one transfer per cycle. In the multi-bus organization, three independent buses allow up to three simultaneous source and destination transfers per cycle, so the same six transfers complete in only two cycles, tripling the efficiency. This shows that multiple buses reduce bus contention and shorten the effective instruction cycle by letting operand fetches and result stores overlap, at the cost of extra bus drivers, control logic, and hardware complexity compared to the simpler but slower single-bus design.

EXPERIMENT 3 – 5-STAGE INSTRUCTION PIPELINE

Aim: To simulate a 5-stage instruction pipeline (Fetch, Decode, Execute, Memory, Write-back), execute a set of instructions, and measure the performance in terms of speedup and throughput compared with a non-pipelined system.

Program:

#include <stdio.h> #define STAGES 5 #define INSTR 5 int main() { char stage[5][4] = {"IF","ID","EX","MEM","WB"}; int totalCycles = INSTR + STAGES - 1; // pipelined execution int nonPipelined = INSTR * STAGES; // non-pipelined execution printf("PIPELINE EXECUTION TABLE (Cycle vs Stage)\n\n"); for (int i = 0; i < INSTR; i++) { printf("I%d: ", i + 1); for (int c = 1; c <= totalCycles; c++) { int s = c - i; if (s >= 1 && s <= STAGES) printf("%s ", stage[s - 1]); else printf("-- "); } printf("\n"); } float speedup = (float)nonPipelined / totalCycles; float throughput = (float)INSTR / totalCycles; printf("\nNon-pipelined cycles = %d\n", nonPipelined); printf("Pipelined cycles = %d\n", totalCycles); printf("Speedup = %.2f\n", speedup); printf("Throughput = %.2f instructions/cycle\n", throughput); return 0; }

Result:

PIPELINE EXECUTION TABLE (Cycle vs Stage) I1: IF ID EX MEM WB -- -- -- -- I2: -- IF ID EX MEM WB -- -- -- I3: -- -- IF ID EX MEM WB -- -- I4: -- -- -- IF ID EX MEM WB -- I5: -- -- -- -- IF ID EX MEM WB Non-pipelined cycles = 25 Pipelined cycles = 9 Speedup = 2.78 Throughput = 0.56 instructions/cycle

Analysis:

The pipeline table shows five instructions overlapping across the Fetch, Decode, Execute, Memory, and Write-back stages, so a new instruction enters the pipeline every cycle once it is full. A non-pipelined processor would need 5 x 5 = 25 cycles to execute the same five instructions sequentially, whereas the pipelined version completes them in only 9 cycles (5 instructions + 4 extra cycles to fill/drain the 5-stage pipeline), giving a speedup of 2.78 and a throughput of 0.56 instructions per cycle. The measured speedup is lower than the ideal 5x because of the pipeline fill and drain overhead; with a larger instruction stream this overhead is amortized and the speedup approaches the number of pipeline stages, confirming that pipelining is most effective for long, hazard-free instruction sequences.

EXPERIMENT 4 – PIPELINE HAZARDS: DATA, CONTROL AND STRUCTURAL

Aim: To develop a pipeline execution scenario where data, control, and structural hazards occur, observe their impact on instruction execution, and implement stalling, data forwarding, and branch prediction to resolve them.

Program:

#include <stdio.h> int main() { /* ---- Data Hazard : ADD R1,R2,R3 ; SUB R4,R1,R5 (RAW on R1) ---- */ int stallsNoFwd = 2; // stalls needed without forwarding int stallsWithFwd = 0; // resolved by data forwarding printf("DATA HAZARD (RAW on R1)\n"); printf("Without forwarding : %d stall cycles\n", stallsNoFwd); printf("With data forwarding: %d stall cycles\n\n", stallsWithFwd); /* ---- Control Hazard : BEQ R1,R2,L1 ---- */ int stallsNoPredict = 2; // flush penalty, no prediction int stallsPredictTaken = 0; // correct prediction int stallsMispredict = 2; // misprediction penalty printf("CONTROL HAZARD (Branch)\n"); printf("No prediction (stall until resolved) : %d cycles lost\n", stallsNoPredict); printf("Branch prediction (correct) : %d cycles lost\n", stallsPredictTaken); printf("Branch prediction (misprediction) : %d cycles lost\n\n", stallsMispredict); /* ---- Structural Hazard : shared memory port for IF and MEM ---- */ int stallsSharedMem = 1; // von Neumann - one port, one stall per conflict int stallsSeparateMem = 0; // Harvard architecture - separate I-mem/D-mem printf("STRUCTURAL HAZARD (Shared Memory Port)\n"); printf("Shared memory (von Neumann) : %d stall cycle per conflict\n", stallsSharedMem); printf("Separate I-mem / D-mem (Harvard): %d stall cycles\n", stallsSeparateMem); return 0; }

Result:

DATA HAZARD (RAW on R1) Without forwarding : 2 stall cycles With data forwarding: 0 stall cycles CONTROL HAZARD (Branch) No prediction (stall until resolved) : 2 cycles lost Branch prediction (correct) : 0 cycles lost Branch prediction (misprediction) : 2 cycles lost STRUCTURAL HAZARD (Shared Memory Port) Shared memory (von Neumann) : 1 stall cycle per conflict Separate I-mem / D-mem (Harvard): 0 stall cycles

Analysis:

The data hazard occurs because the SUB instruction needs R1 in its Decode stage before the preceding ADD has written it back, causing two stall cycles if the processor simply waits; forwarding the ALU result from the EX/MEM latch directly to the next instruction's input removes the hazard entirely. The control hazard arises because the outcome of a branch is not known until later in the pipeline, so instructions fetched after it may be wrong; without prediction the pipeline loses two cycles every time, whereas branch prediction avoids the loss on a correct guess and only pays the same two-cycle penalty on a misprediction, giving a net performance gain whenever prediction accuracy is reasonably high. The structural hazard occurs because instruction fetch and data memory access compete for a single memory port in a von-Neumann-style design; using separate instruction and data memories (Harvard architecture) removes the conflict completely and eliminates the associated stall cycle.

EXPERIMENT 5 – SUPERSCALAR, OUT-OF-ORDER, SPECULATIVE EXECUTION AND SMT

Aim: To analyze a processor supporting superscalar execution, including out-of-order and speculative execution, extended to hyper-threading and simultaneous multithreading (SMT), and to evaluate the resulting instruction throughput and resource utilization.

Program:

#include <stdio.h> int main() { int instructions = 8; int scalarCycles = instructions; // 1-issue scalar int superscalarCycles = (instructions + 1) / 2; // 2-issue, ideal int oooStallCycles = 1; // dependency stall (in-order) int superscalarOOOCycl = superscalarCycles; // OOO hides the stall printf("Scalar (1-issue) execution: %d cycles\n", scalarCycles); printf("Superscalar (2-issue, in-order): %d cycles (+%d stall)\n", superscalarCycles + oooStallCycles, oooStallCycles); printf("Superscalar (2-issue, out-of-order + speculative): %d cycles\n\n", superscalarOOOCycl); float speedupSS = (float)scalarCycles / (superscalarCycles + oooStallCycles); float speedupOOO = (float)scalarCycles / superscalarOOOCycl; printf("Speedup (in-order superscalar) = %.2f\n", speedupSS); printf("Speedup (out-of-order superscalar) = %.2f\n\n", speedupOOO); /* Simultaneous Multithreading : 2 threads share one superscalar core */ int utilSingleThread = 50; // % issue-slot utilization, 1 thread int utilSMT = 95; // % issue-slot utilization, SMT fills idle slots printf("Simultaneous Multithreading (SMT), 2 threads\n"); printf("Issue-slot utilization (single thread) = %d%%\n", utilSingleThread); printf("Issue-slot utilization (SMT, 2 threads) = %d%%\n", utilSMT); return 0; }

Result:

Scalar (1-issue) execution: 8 cycles Superscalar (2-issue, in-order): 5 cycles (+1 stall) Superscalar (2-issue, out-of-order + speculative): 4 cycles Speedup (in-order superscalar) = 1.60 Speedup (out-of-order superscalar) = 2.00 Simultaneous Multithreading (SMT), 2 threads Issue-slot utilization (single thread) = 50% Issue-slot utilization (SMT, 2 threads) = 95%

Analysis:

Superscalar execution issues two instructions per cycle instead of one, so an in-order 2-issue processor already improves execution from 8 cycles to 5 (plus one unavoidable dependency stall), a 1.60x speedup. Allowing out-of-order execution lets the processor issue a later, independent instruction while the dependent one waits, hiding the stall entirely and raising the speedup to 2.00x; this is supported by a reorder buffer and reservation stations that track true data dependencies while permitting speculative execution past predicted branches, with results committed only if the speculation proves correct. Extending the core with simultaneous multithreading lets a second hardware thread's instructions fill the issue slots left idle by the first thread's dependencies, raising issue-slot utilization from about 50% to 95%; overall system throughput therefore increases substantially even though the latency of any single thread is not improved, illustrating why SMT is used to exploit instruction-level parallelism that a single thread alone cannot supply.